Problem Name : Write a code to perform frequency modulation and de-modulation using MATLAB function.

Sourse Code:

clc;

clear all;

close all;

fs = input('Enter Sample Frequency:');

fc = input('Enter Carrier Frequency:') ;

t = (0:1/fs:0.2);

x = sin(2*pi*30*t);

fdev = input('Deviation Frequency:') ;

y = fmmod(x, fc, fs, fdev);

z = fmdemod(y, fc, fs, fdev);

subplot(3, 1, 1);

plot(t, x);

title('Message Signal');

subplot(3, 1, 2);

plot(t, y);

title('Modulated Signal');

subplot(3, 1, 3);

plot(t, z);

title('Demodulated Signal');

Input:

Output:

Problem Name : Write a code to perform Phase modulation and de-modulation using MATLAB function.

Sourse Code:

clc;

clear all;

close all;

fs = input('Enter Sample Frequency:');

t = 0:1/fs:3;

fc =input('Enter Carrier Frequency:') ;

fm =input('Enter Message Frequency:') ;

pdev = pi/2 ;

ct = 1*sin(4*pi*fm*t);

mt = 1*sin(2*pi*fm*t)+ct;

x = pmmod(mt, fc, fs, pdev);

y = pmdemod(x, fc, fs, pdev);

subplot(4, 1, 1);

plot(t, mt, 'b');

title('Message Signal');

subplot(4, 1, 2);

plot(t, ct, 'r');

title('Carrier Signal');

subplot(4, 1, 3);

plot(t, x, 'r');

title('Phase Modulation Signal');

subplot(4, 1, 4);

plot(t, y, 'm');

title('Phase Demodulation Signal');

Input:

Output:

Name : Write a code to perform frequency modulation and de-modulation using MATLAB Programming.

Sourse Code:

clc;

clear all;

close all;

t = [0:0.001:1];

f1 = input('Enter Frequency1:');

m = cos(2*pi*f1*t);

subplot(4, 1, 1);

plot(t, m);

title('Message Signal');

f2 =input('Enter Frequency2:') ;

c = sin(2*pi*f2*t);

subplot(4, 1, 2);

plot(t, c);

title('Carrier Signal');

mf = input('Message Frequency:');

s = sin((2*pi*f2*t)+(mf*sin(2*pi*f1*t)));

subplot(4, 1, 3);

plot(t, s);

title('FM signal');

x = diff(s);

y = abs(x);

[b, a] = butter(10, 0.056);

s1 = filter(b, a, y);

subplot(4, 1, 4);

plot(s1);

title('Demodulation');

Input:

Output:

Problem Name : Write a code to perform phase

modulation and de-modulation using MATLAB Programming.

Sourse Code:

clc;

clear all;

close all;

clearvars;

fc = input('Enter Carrier Frequency:');

fm =input('Enter Message Frequency:') ;

alpha = 1;

theta = pi/4;

beta = pi/5;

receiverKnowsCarrier= 'False';

fs = 8*fc;

duration = 0.5;

t = 0:1/fs:1-1/fs;

m_t = alpha*sin(2*pi*fm*t + theta);

c_t = alpha*sin(2*pi*fc*t + beta);

x = cos(2*pi*fc*t + beta + m_t );

figure(); subplot(4,1,1)

plot(t,m_t)

title('Modulating signal'); xlabel('t'); ylabel('m(t)')

subplot(4, 1, 2)

plot(t, c_t)

title('Carrier signal'); xlabel('t'); ylabel('c(t)');

subplot(4,1,3)

plot(t,x)

title('Modulated signal'); xlabel('t');ylabel('x(t)')

nMean = 0;

nSigma = 0.1;

n = nMean + nSigma*randn(size(t));

r = x + n;

z= hilbert(r);

inst_phase = unwrap(angle(z));

if strcmpi(receiverKnowsCarrier,'True')

offsetTerm = 2*pi*fc*t+beta;

else

p = polyfit(t,inst_phase,1);

estimated = polyval(p,t);

offsetTerm = estimated;

end

demodulated = inst_phase - offsetTerm;

subplot(4,1,4);

plot(t,demodulated);

title('Demodulated signal'); xlabel('n'); ylabel('\hat{m(t)}');

Input:

Output:

Problem Name : Write a code to perform Frequency Shift Keying using Matlab Programming.

Sourse Code:

clc;

close all;

clear all;

ac = input('Enter Carrier Amplitude:');

fc = input('Enter Carrier Frequency:') ;

b = input('Enter the number of bits: ');

bs = randi([0 1], 1, b);

t = 0.01:0.01:b;

for i=1:b

mt((i-1)*100+1:i*100) = bs(i);

end

ct = ac*cos(2*pi*fc.*t);

st = ac*cos(2*pi*fc*(mt+1).*t);

subplot(3, 1, 1);

plot(t, mt);

title('Modulating Signal');

grid on;

subplot(3, 1, 2);

plot(t, ct);

title('Carrier Signal');

grid on;

subplot(3, 1, 3);

plot(t, st);

title('FSK signal');

grid on;

Input:

Output:

Problem Name : Write a code to perform Frequency Shift Keying using Matlab Simulink Model.

Output:

Problem Name : Write a code to perform Phase Shift Keying using Matlab Programming.

Sourse Code:

clc;

close all;

clear all;

ac = input('Enter Carrier Amplitude:');

fc =input('Enter Carrier Frequency:') ;

b = input('Enter the number of bits: ');

bs = randi([-1 1], 1, b);

t = 0.01:0.01:b;

for i=1:b

mt((i-1)*100+1:i*100) = bs(i);

end

ct = ac*cos(2*pi*fc.*t);

st = mt.*ct;

subplot(3, 1, 1);

plot(t, mt);

title('Modulating Signal');

grid on;

subplot(3, 1, 2);

plot(t, ct);

title('Carrier Signal');

grid on;

subplot(3, 1, 3);

plot(t, st);

title('PSK signal');

grid on;

Input:

Output:

Problem Name: write a code to perform phase shift keying using Matlab simulink Model.

Output:

Problem Name : Write a code to perform quantization of an analog signal using MATLAB.

Sourse Code:

clc;

clear all;

close all;

Ts=0.0001;

t=0:.00001:20*Ts;

sig=sin(2000*pi*t)+cos(2000*pi*t);

maxsig=max(sig);

interv=2*maxsig/(2^3-1);

u=maxsig+interv;

partition = [-maxsig:interv:maxsig];

codebook = [-maxsig:interv:u];

[index,quants] = quantiz(sig,partition,codebook);

subplot(2, 1, 1);

plot(t,sig,'-');

title('Original Signal');

subplot(2, 1, 2);

plot(t, quants, '-');

title('Quantized Signal');

Output:

Problem Name: Write a code to perform amplitude modulation using MATLAB Simulink Model.

Output:

Problem Name: Write a code to perform amplitude de-modulation using MATLAB Simulink Model.

Output:

Problem Name: Write a code to perform frequency

modulation using MATLAB Simulink Model.

Output:

Problem Name: Write a code to perform frequency de-modulation using MATLAB Simulink Model.

Output:

Problem Name: Write a code to perform phase modulation using MATLAB Simulink Model.

Outout:

Problem Name: Write a code to perform Phase de-modulation using MATLAB Simulink Model.

Output: